Checklist for Waterbodies Related Development Application

Proposal: _____________________________________________________________________________________________

_____________________________________________________________________________________________

| 1 | SUBMISSION PLANS | REMARK (Please indicate File Name) |
| --- | --- | --- |
| 1.1 | Site Plan / Location Plan to show the Width and Alignment of Drainage Reserve/ Waterbody/ Coastal Profile. | File Name: |
| 1.2 | Cross-sections showing the area between the site and the Drainage Reserve/ Waterbody/ Coastal Area including the existing and proposed ground levels. | File Name: |
| 1.3 | Details of boundary wall / fencing (Height, Type, Screening if any). | File Name: |
| 1.4 | Contour lines extending to 1m beyond the side of development fronting the Drainage Reserve/ Waterbody/ Sea. | File Name: |

| 2 | COMPLIANCE CHECK | Yes | No | NA | Remark |
| --- | --- | --- | --- | --- | --- |
| 2.1 | SLOPE a) There shall be no abrupt change in slope at boundary between the development and the drainage reserve (max. gradient 1:2). |  |  |  | If no, please state gradient. |
| 2.2 | RETAINING WALL a) Retaining walls are < 1.5m in height. b) Retaining walls are set back at least 300mm from the boundary. c) Retaining walls are covered with creepers. |  |  |  | If no, please provide justifications. |
| 2.3 | BOUNDARY WALL (For Residential, Commercial, Institution, School and Recreational Club Developments) a) The proposed fencing affords visibility through the fencing. E.g. Chained-link, metal-grille or timber fence. (For Factories, Warehouses, Storage Yards) b) The boundary wall is a solid-type wall of less than 1.8 m high; or a chain-linked type of less than 1.8m high with close planting of shrubs / vegetation as screen and provision of 0.5m wide planting strip within the development boundary. |  |  |  | If no, please provide design & choice of materials and explanation. |
| 2.4 | Mature vegetation are saved and conserved. |  |  |  |  |

*I / We certify that the above information given in this application is true and correct.

_________________________________________________________________                                                 ______________

Name and Signature of Qualified Person (QP)                                                                                                                Date